Instructions for converting .mq analysis files to .an format.

Use of fcheck, harmformat, and gohfan, gomqhfan, and gomqhfan1.

Conversion of MQ analysis files to .an format requires two steps: First, the

fundamental frequency of the analyzed signal is determined using the 'fcheck'

program. Next, the MQ and fundamental frequency data are used to extract

quasi-harmonic partials for conversion to .an format using the program

'harmformat'. The combination of these steps can be automated using the

script 'gohfan'.

The sequence, MQ-analysis, ff extraction, and harmonic format production, can

be automated using the scripts 'gomqhfan' and 'gomqhfan1'. The difference

between the two is that the latter includes generation of a "header file",

whereas the latter assumes this file has already been created.

Procedure:

The 'fcheck' program is designed for determining the pair of fundamental

frequencies in a musical duet. For converting MQ to monan, the 'fcheck' program

is used to identify a the fundamental frequency of a monophonic signal so some

of the command line parameters do not apply.  The current version of this

program assumes that the MQ files has a header on it which supplies basic data

about the original sound file as while as analysis data.

Usage:

fcheck file.mq frame0 minfl maxfl minfh maxfh n0 n1 minlev low.ffh high.ffh

where     file.mq   is the MQ spectrum peak file with header.

frame0     is the starting frame number (usually zero).

minfl and

maxfl      are the minimum and maximum frequencies (in Hz)

of the lower voice for the fundamental frequency

search.

minfh and

maxfh      are the minimum and maximum frequencies (in Hz)

of the upper voice for the fundamental frequency

search. (both 0 if monophonic)

n0         is the number of harmonics to use in determining

the frequency of the lower voice.

n1         is the number of harmonics to use in determining

the frequency of the upper voice. (0 if mono)

minlev     is the minimum level (in amplitude dB) for

MQ peaks used in calculating the frequency.

low.ffh    is the output file containing the fundamental

frequency estimates for the lower voice (with

header.

high.ffh   is the output file containing the fundamental

frequency estimates for the upper voice (with

header).

fcheck/harmformat						2

monophonic example:

fcheck file.mq 0 100 130 0 0 20 0 0 file.ffh /dev/null

This will input the MQ file "file.mq", determine an estimated fundamental

frequency in the range 100 to 130 Hz beginning with the first frame, use up to

20 partials, and write to the frequency file "file.ffh". NOTE that the command

line arguments referring to the upper voice are specified as zeros, and the

upper voice output file is ignored (writes to /dev/null).

Once the fcheck process is complete, harmformat is used to perform the

quasi-harmonic extraction.

Usage:

harmformat file.ffh file.mq file.mq.an dev emphasis

where   file.ffh 	is a float data file containing fund. freq. vs.

time data (with header).

file.mq 	is a float data file containing spectral peak

information (with header).

file.mq.an is the output file containing

harmonic amplitude and frequency data vs. time.

dev 		is the fraction of departure from harmonicity

allowed for a peak component to considered as a

harmonic. (typical value: .03)

emphasis 	emp=1 means that high frequency emphasis was

applied in the MQ analysis and this must be

corrected for.

Here is a typical gomqhf interaction:

% gohfan

sound file name (without .snd extension) is: guitar

search range minimum fund. freq: 90

search range maximum fund. freq: 130

monan analysis frequency (fundamental): 110

Number of partials to retain: 80

amplitude threshold for analysis is (in range of -20 to 10): -10

harmonic acceptance interval (in range of .01 to .1): .03

type: tail -f guitar.mqhf.list  to check on job progress

[1] 12014

% tail -f guitar.mqhf.list

gohfan automatically picks up files guitar.mq and guitar.mq.head, creates

files guitar.ff and guitar.mq.an, and redirects to the file guitar.mqhf.list.

For fcheck the number of harmonics used for pitch detection (n0) is set to 10

and n1 is set to 0 (monaural case). For harmformat hai and emphflag are

defaulted to .03 and 1, respectively.

Scripts 'gomqhfan' and 'gomqhfan1' combine the operations of  gomqan(1),

fcheck, and harmformat.  For descriptions of their use, see the file mqan.doc.

_______________________________________________________________________________

Rob Maher                                            May 29, 1989

Revised: J. W. Beauchamp 11/24/95