Monan: A Spectral Analysis Processing Program

0. Introduction

monan is a comprehensive program for display, modification, and synthesis of

spectrum analysis data obtained from files containing analysis data (*.an

files). Special commands are available for accomplishing various tasks of these

types. monan runs under the NeXTStep Terminal application and uses the Preview

EPS graphics viewer, or for remote login to a NeXT it uses Tektronix 4014

graphics.  Under XWindows it uses ghostview or xpsview for graphics.

1. Definitions

2. Using Monan

Enter the terminal mode and have an analysis (.an) file available in the

current directory. Type monan <return>. The program will prompt you for the name

of the analysis file and the number of harmonics you wish to read in. You can

hit <return> to get the default or maximum number of harmonics available. You

will then be prompted for a command with

Give command(type 'lc' for command menu)

After giving and completing a proper command, the prompt will be

next?

However, if you give an invalid command or just hit <return> will get

huh? type 'lc' for menu

next?

Here is a list of the (mostly) two-character commands available in monan:

2.1 Plot commands:

aa--Plot harmonic k1 amplitude vs. harmonic k2 amplitude.

Continuous plot of a particular harmonic amplitude vs. another harmonic

amplitude. You give the desired two harmonics and the time range of the plot.

Harmonic 0 refers to the rms amplitude. Giving a negative harmonic no. allows

you to specify a range of harmonics to be plotted in overlay fashion. Amplitude

can be either linear or decibel. These types of plots are useful for revealing

correlations between harmonic amplitudes with time as a parameter.

ak--Plot or list harmonic amplitude vs. harmonic number.

A listing or bar graph of harmonic amplitude vs. harmonic number (spectrum) at

a particular time or averaged over the duration of the tone. Amplitudes are on

either a linear or decibel scale. Frequencies can be spaced either in linear or

logarithmic fashion, but are always multiples of the analysis fundamental

(base) frequency fa (specified when using gopvan or gomqhfan to produce an

analysis file). You give a "snapshot" time (in seconds) corresponding to the

frame to be graphed. (This time should be within the duration of the sound.) or

'-1' if you want an average spectrum over the sound's duration. You can limit

the maximum number of harmonics to be displayed. The horizontal scale can be

either "harmonic number" or "frequency".

af--Plot harmonic amplitude vs. frequency superimposed for several harmonics.

For each harmonic its amplitude is plotted vs. its frequency in overlay

fashion. This plot is useful for revealing formant structures, particularly if

substantial frequency variations occur during the sound.

at--Plot harmonic (or rms) amplitude vs. time.

Continuous plot of amplitude (envelope) vs. time for a specified harmonic. You

give the desired harmonic and the time range of the plot. The amplitude scale

can be either linear or decibel. Harmonic 0 specifies that an overall (rms)

amplitude envelope plot is desired. If the amplitude scale is linear there is

an option for normalizing the amplitudes by the rms amplitude (relative

amplitude plot).

br--Plot normalized spectral centroid (brightness) vs. rms amplitude.

For each point in time the normalized spectral centroid is defined as the ratio

of the sum of harmonic amplitudes each weighted by their harmonic numbers

divided by the simple sum of harmonic amplitudes.  I.e.,

BR(t) = SUM(k=1,k=nhar){k*A[k](t)}/SUM(k=1,k=nhar){A[k](t)}.

BR(t) is a measure of perceptual brightness of a sound as it varies with time.

Here time is a parameter (you give a range of times for the plot), and

amplitude is the independent variable. This plot is useful for determining

whether there is a strong correlation between spectral centroid and rms

amplitude.

bt--Plot normalized spectral centroid (brightness) vs. time.

BR(t) as defined under 'br' is plotted as a function time.

cr--Plot spectral centroid (as defined under ct) vs. the rms amplitude. Also

gives the coefficients for a straight line approximation to the data.

ct--Plot spectral centroid vs. time.

Spectral centroid defined as

fc(t) = fa*(BR(t) - 1)

is graphed as a function of time.

fk--List average frequency deviation vs. harmonic number.

Average frequency deviations and standard deviations are given for each

harmonic. This tells you how close the actual frequencies are to the expected

harmonic frequencies assumed in the analysis stage. If they are not close, you

might want to repeat the analysis. At the end of the printout, an average

analysis frequency is given which can be used to repeat the analysis in order

to achieve a more accurate result.

Here is what the various listed designations mean:

HARM is the harmonic number (wt. ave refers to the weighted average of the

first 5 harmonics; see description under 'ft'.)

NFD is FD divided by the analysis frequency times the harmonic number.

RMSD is the standard (rms) deviation of the frequency from its average value.

NRMSD is RMSD divided by the analysis frequency times the harmonic number.

PEAK is the maximum absolute deviation in Hz.

NPEAK is PEAK divided by analysis frequency times the harmonic number.

fi--Plot inharmonicity of harmonic k vs. time.

The existence of non-tracking partials or inharmonicity between the partials

can be tested using this command. The weighted average frequency deviation

is subtracted from the deviation of a given harmonic. I.e.,

(dfr[k]/k - dfr.wt.ave)/fa. If the result is zero, there is no inharmonicity.

The degree of inharmonicity is indicated by this difference. However, there may

be very small differences due to signal noise and calculation errors.

ft--frequency or frequency deviation for a particular harmonic (or weighted

average) is graphed as a function of time.

Three plot type options are available: (f) frequency of a harmonic, i.e., fk,

in Hz vs. time. (n) normalized frequency deviation, i.e., (fk - k*fa)/(k*fa),

vs. time. (c) cents deviation, i.e., 1200*log(fk/(k*fa))/log(2), vs. time. fk

is the instantaneous frequency of harmonic k. The user gives the time over

which the graph is plotted. If all harmonics are tracking one another in

frequency, i.e., fk(t) = k*f1(t), the normalized and cents ft graphs for all

harmonics should be identical.

Let's say that the frequency of harmonic 5 varies between 490 and 510 Hz and

that fa = 100 Hz (the analysis frequency specified in the analysis file's

header). After a plot type option is specified the user is asked to give the

vertical axis frequency range for the plot. For this case, if the user had

specified option (f), an appropriate vertical range would be '490 510'.

(Defaults correspond to +- .5*fa.) If he asked for option (n), a good vertical

value would be .02, since the deviation is never greater than 10 Hz

corresponding to a normalized value of 10/(5*100). The graph will actually

extend from -.02 to +.02. In the case of option (c), 40 would be a good

vertical value, since the graph would extend from -35.0 to +34.2 cents.

Harmonic 0 gives the instantaneous weighted average frequency deviation based

on the first 5 harmonics. Each harmonic's deviation is weighted according to

its instaneous amplitude. This is defined by

df/f.wt.ave = SUM(k=1,5){A[k]*dfr[k]/(k*fa)}/SUM(k=1,5){A[k]},

where k=harmonic no., A[k] amplitude of kth harmonic, dfr[k] = frequency

deviation of kth harmonic, and fa = analysis frequency.

ftc--Plot composite of all harmonic frequencies vs. time.

Vertical axis is frequency, and horizontal axis is time. User can give range of

harmonics to be plotted and optionally can have each frequency graph windowed

by +-x*fa, where x is an arbitary value. Also, the user can designate an

amplitude threshold below which frequencies are not graphed.

pp--Plot harmonic amplitude vs. harmonic number and time.

This command results in a composite plot of amplitude vs. time for several

harmonics in succession. As such, it is a three dimensional representation

("perspective plot") of the time-variant spectrum of a sound. Amplitude,

harmonic number, and time scales are linear.

pt--Plot "musical pitch" vs. time.

Pitch is defined as the log of the weighted average frequency. The vertical

scale is given in "note values", i.e., C4, F#5, and a whole number of octaves

are displayed. Horizontal axis is time in seconds. The user is prompted for

1) the beginning and ending times of the plot, 2) whether to override the

automatically determined pitch range, 3) an amplitude threshold, and 4) an

allowable pitch change between analysis frames. The last two items are used to

eliminate "glitches" from the plots. If you give an amplitude threshold of,

say, 100, pitches corresponding to amplitudes below 100 will not plot. If you

give an allowable pitch change of, say, 3 semitones, pitch changes greater than

3 between two adjacent frames will not plot.

si--Plot "spectral irregularity" vs. time

Spectral irregularity is defined as the average absolute difference between

a harmonic amplitude and the average of itself and its two neighbors. If a

spectrum is very smooth -- i.e., it follows a straight line pattern, its

irregularity is zero. In the averaging process, components are weighted by the

amplitude of the central component. Also, the average is normalized by the rms

amplitude, so that the result is the "average fractional spectral

irregularity". Thus, the spectral irregularity is defined

as

SUM(k=2,nhar-1) A[k]*abs((2./3.)*A[k] - (1./3.)*(A[k-1] + A[k+1]))

SIR = -------------------------------------------------------------------

Arms*SUM(k=2,nhar-1) A[k]

where SIR, A[k] (the kth harmonic amplitude), and Arms (see rm command) are

functions of time.

2.2.  Modification Commands

fa--replace harmonic frequency deviations with average values (fixed

inharmonics).

fc--replace each harmonic frequency deviation with the df/f.wt.ave composite

frequency deviation scaled by the harmonic number. All harmonics are in strict

harmonic relationship even though the frequency is time-varying.

ff--replace each harmonic frequency deviation with the deviation of a

designated harmonic scaled by k/k0, where k is the current harmonic number and

k0 is the designated harmonic number.

fh--replace harmonic frequency deviations with zero values (fixed harmonics).

fv--replace the harmonic frequency deviations with a sinusoidal variation such

that

df1(t) = vib_depth*fa*sin(2*PI*fv*t)

and	dfk(t) = k*df1(t)

where vib_depth is the vibrato depth and fv is the vibrato frequency (rate) in

Hz.

ri--Replace each harmonic amplitude by the linear average of itself and its

two (original) neighbors. Since spectral irregularity is the average absolute

difference between components and their neighbors, this operation reduces the

spectral irregularity. All time frames are processed, and each time frame is

processed separately.

rm--Replace individual harmonic amplitude curves by rms envelopes which are

scaled according to the average values of each harmonic amplitude.  This tests

the importance of having individual harmonic envelopes while retaining the

overall amplitude vs. time and average spectral structure of the sound. First

the Arms(t) is computed as

Arms(t) = sqrt(SUM(k=1,k=nhar){A[k](t)*A[k](t)})

for each time frame. Then, the average A[k] values are computed as

A[k]ave = SUM(t=0.1*tl,t=0.9*tl){A[k](t)}/tl,

and the average rms as

rms_ave = SUM(t=0.1*tl,t=0.9*tl){rms(t)}/tl,

where tl is the time length of the sound.  Finally, the new harmonic amplitudes

are computed in terms of the old values according to

A'[k](t) = A[k]ave*rms(t)/rms_ave.

With this method, the average values of the individual harmonic amplitudes are

preserved.  The rms amplitude is preserved within a gain factor which is

generally close to unity.

rd--Reduce the duration of the amplitude and frequency envelopes by

interpolating the "steady-state" portion of the sound. Designated intervals

(0,t1) and (t2,tL) (corresponding to the "attack" and "decay", respectively)

are unchanged. (tL is the original duration and dur is the new duration.)

However, the interval (t1,t2-(tL-dur)) is cross-faded with the

interval (t1+tL-dur),t2) in order to shorten this "steady-state" portion.

sa--Scale harmonic amplitudes in order to achieve a specified spectrum at a

specified time. This is useful for creating "hybrid sounds". Upon resynthesis

the hybrid sound has the harmonic envelope shapes of the original sound but

with amplitude weights according to the given target spectrum. Spectra are

scaled so that the original rms amplitude is preserved at the specified time.

sm--smooth (apply low pass filter to) harmonic amplitudes or frequencies. You

give a low pass filter cutoff frequency and an offset time (from time=0) where

the smoothing is to begin.

c.  I/O Commands

as--read n-segment harmonic amplitude or normalized frequency deviation file

The data file consists of a series of coordinate breakpoints which are linearly

interpolated to replace the current data. For each harmonic k the data is

in the form of

k  n  t[1]  c[k,t[1]]  . . .  t[n]  c[k,t[n]]

where k is the harmonic number, n is the number of coordinates, c can be cmag

or dfr/(k*fa) values, and t[j], c[k,t[j]] is the time/value coordinate of the

jth breakpoint for the kth harmonic.

ih--read n-segment interharmonic amplitude relationships file

The data file consists of interharmonic relationship n-vectors given as a

series of breakpoints for each harmonic. Given the harmonic number i on the

first line, for each harmonic k, the data is given by

k  n  c[i,1] c[k,1] . . .  c[i,n] c[i,n]

where k is the harmonic number, n is the number of coordinates, and the pair

c[i,j], c[k,j] (where j is the coordinate number) represents the cmag[k] vs.

cmag[i] coordinate for each harmonic. "i" is the "driver" harmonic number. This

is because the original data for the ith harmonic is retained, and the other

harmonic amplitudes are regenerated based on the nonlinear curves given by

these data.

rf--read new analysis file

With this command, a new analysis file is read in (as in the beginning) and all

of the previous analysis data is overwritten. The user can read all or some of

the analysis file's harmonics.

sp--play sound file

This allows the user to play a sound file in the current area without exiting

monan.

sy--synthesize sound file

A sound file is created using additive synthesis based on the time-variant data

currently in memory. This is useful for exploring the result of various data

modifications. Options allow the user to change the sample rate, the

fundamental frequency, the range of harmonics, the amplitude factor, and the

duration of the output file. The duration is scaled uniformly over the sound.

wf--write analysis file based on current (possibly modified) analysis data.

This writes a new analysis file based on the current analysis data in memory.

Thus, if you have modified the data (for instance, by using the 'sm' command),

the modified version of this data will go into the new analysis file. Also, you

have the opportunity to limit the number of harmonics which are written into

the output file. This could be as low as zero harmonics. When zero harmonics

are specified, the rms and dfr.wt.ave vs. time parameters are written to the

file, but no actual harmonics. This is useful for subsequent pitch plotting

since it keeps the analysis files only as they need to be for this application.

The wf command writes the analysis data as a 'compact' file, which takes up

half the space of the 'simple' or 'compressed' file produced by gopvan or

gomqhfan.  (Files in the 'simple' or 'compressed' formats can also be converted

to 'compact' format using the standalone program 'tocompact'.)

d. Miscellaneous Commands

ex--exit monan.

EPS files in /tmp are destroyed by this command.

lc--list command menu.

This gives a summary of all of the monan commands.

lh--list current filename and header information.

The following information about the current analysis file is given: file name,

instrument name, pitch, dynamic, analysis frequency, duration (in seconds),

time for each frame (in sec), total number of frames, number of harmonics, file

format, date recorded, and comments.

lv--list amplitude or frequency values. You can use this command to get the

exact value of an amplitude or frequency (absolute, deviation, or normalized

deviation) for a given harmonic and at a particular time value.

mh--modify header data

After typing the 'mh' command, each header datum is given one at a time. If you

don't like the value or string given, you can subsititute your own version;

otherwise just hit <return>. This changes the internal representation of the

analysis file header but not the file header itself. To update the file, you

can use the 'wf' command to replace the current version of the file.

re--toggle research mode

This command allows you to control the numbers that are labled on the

horizontal and vertical axes. This is helpful in situations where the program

does not give appropriate numerical scale values. Since normally monan is

not in research mode, you have to to type 're' to get into this mode. (Type

're' again, if you want to return to normal mode.) When in research mode, the

program gives values of x_min, x_max, x_increment, and no_of_tick_marks, for

each axis, and asks if you approve of these values. If you don't, you can

substitute your own values.

-----------------------------------------------------------------------------

James Beauchamp, 01/18/96