PVAN, GOPVAN, GOPVAN1, MAKEHEADER

NAMES: pvan, gopvan, gopvan1, makeheader

USE:  perform pv analysis of sound file (.fp, .sh., or .snd format)

DESCRIPTION:

These programs combine to perform phase vocoder analysis of an input sound

sample file and produce an output file suitable for examination, modification,

and resynthesis using the graphics program monan. Result is a time-variant

harmonic analysis of the input sound file, where amplitude (magnitude) and

frequency deviation (from mean harmonic values) are stored for sequential time

frames. .fp and .sh files are headerless files consisting of floats or short

ints, respectively. A .snd file is a file of short int samples with a standard

NeXT header.

gopvan is a shell script (which calls pvan and makeheader) which first prompts

the user for salient information about the input file and analysis parameters,

and then launches pvan as a background job. gopvan1 is similar except it uses

an already existing header file (created by a previous gopvan job).

gopvan1 works the same way as gopvan except that it assumes that the header

file has already been created from a previous analysis attempt. This is useful

if, for example, the user wishes to change the analysis frequency in order to

reduce the error between the sound's actual fundamental and the analysis

frequency (as revealed by pvan's listing of f1 vs. time or by monan's fk

command).

pvan is the binary executable version of the analysis program, which can be

run as follows:

pvan <analysis freq> <header file> <sound file> <analysis file> >& <diagnostics listing file>

The analysis output file consists of a header block and starting phase data,

followed by magnitude/frequency data frames measured every half-cycle of the

fundamental frequency of the input data file. The header information is stored

in a separate file to facilitate running pvan, and this file may be kept for

subsequent runs of the analysis. The number of harmonics saved in the file is

equal to the signal sample rate divided by twice the analysis frequency.

When using gopvan, the user supplies the following information as he is

prompted:

- The name of the sound file name sans the .snd extension, which is assumed,

i.e., trumpet for trumpet.snd.  Then the header, analysis,

and diagnostics list files will be trumpet.pv.head, trumpet.pv.an, and

and trumpet.pv.list, respectively.)

- The analysis (fundamental) frequency in Hz.

- Data for the header (gopvan only) as follows:

- The name of the performer.

- The type of instrument used (flute, etc).

- The date of the recording (06/19/78, etc.).

- The musical pitch played. (Bb4, G#3, etc.)

- The musical dynamic level.(mf, pp, etc.)

- Vibrato? (YES or NO).

- Tone segment (all, middle, or end).

- Further comments on sound sample (history, instrument manufacturer, etc.)

pvan, gopvan, gopvan1, makeheader	 2

makeheader is a program, used within gopvan and gopvan1, which prompts the user

for the musicological information listed above and outputs a headerfile. It can

also be used standalone with command line arguments as follows:

makeheader <headfile> <performer> <instrument> <date> <pitch> <vibrato> \

<part> <comments>

Note that any strings which contain spaces must be enclosed in quotes.

THEORY OF OPERATION

First, bandlimited interpolation is performed on the signal to adjust the

sample rate of the signal so that there are 2^M samples per fundamental

period, which is the inverse of the given fundamental frequency (fa), where

M = log2(ceil(SR/f1)). The signal is then windowed by a Hamming window and an

FFT is computed over a double period. Magnitude and phase for each FFT spectrum

component is then computed, and the frequency for each component is taken as

the difference between the phases of adjacent frames divided by 2*pi times the

interframe time, which is a half-period. For the normal "simple" format only

the even spectral components are kept, which correspond to the harmonics of the

signal. For each frame the magnitude and frequency for each of SR/(2*fa) -1

harmonics are written to the output file.

RELATED FILES

gopvan*       	shell script to launch job using makeheader and pvan

gopvan1*	shell script to launch job using existing header file

makeheader*    	prompt program to create analysis file header and header file

pvan*    	pv analysis program

monan*		analysis viewing/modification program

DIAGNOSTICS

Reports any file I/O errors and ends processing.

LIMITATIONS

There is no way to examine the analysis output as the process is running.

However, you can do

tail -f <list file>

to check on progress. This gives a list of frame numbers, times and amplitudes

and frequency deviations of the first harmonic.  The frequency deviations

should be small to give good results.  This file will end with "Analysis

completed ... " when the analysis job is done.

The program relies on the user to supply the analysis frequency. This could be

the frequency corresponding to the pitch the sound is "known to have".

Alternatively, the fundamental frequency can be determined by other means, such

as by using the command 'gomqff' to produce an '.ffh' file which can be listed

using the 'fd' (frequency dump) command.

Revised: J.W. Beauchamp 11/12/96